IEEE Standards (THE IEEE 802 FAMILY)

Origin of the 802 Number

The IEEE decided to name the 802 family set of standards “802” because it was in February of 1980 that they started this project of standardization.

The Institute of Electrical and Electronic Engineers (IEEE) is one of the largest professional organizations in the world, and is extremely influential with regard to setting standards. In February of 1980, the IEEE  implemented a task force to develop a set of standards for connectivity between Network Interface Cards (NICs) and transmission media. This task force was known as the 802 committee. This 802 committee was broken down into several different subcommittees that were each responsible for some different implementation of data transfer that occurs at the Data Link level of the OSI model. These IEEE standards have also been adopted by ISO, and they are referred to as ISO 8802. Thirteen workgroups oversee the 802 standards. Each workgroup is assigned a specific mandate in the area of LAN/MAN connectivity they are to analyze. Figure 1. illustrates the position each standard occupies in the OSI reference model.

FIGURE 1: The relationship between the IEEE 802 standards and the OSI model.

IEEE 802.1

This standard is actually one that goes beyond the Data Link layer of the OSI model. This is a general standard for network management, and provides network management standards to the other 802 standards in the OSI model. This standard actually covers all layers from the Physical to the Transport layer.

IEEE 802.2

The IEEE 802.2 standard defines an LLC sublayer that is used by other lower-layer protocols. Because these lower-layer protocols can use a single LLC protocol layer, Network layer protocols can be designed independently of both the network’s Physical layer and MAC sublayer implementations. The LLC appends to packets a header that identifies the upper-layer protocols associated with the frame. This header also declares the processes that are the source and destination of each packet. The workgroup for this set of standards is currently inactive.

IEEE 802.3 (Ethernet Network)

The IEEE 802.3 standard defines a network derived from the ethernet network originally developed by Digital, Intel, and Xerox. This standard defines characteristics related to the MAC sublayer of the Data Link layer and the OSI Physical layer. With one minor distinction—frame type—IEEE 802.3 Ethernet functions identically to

DIX Ethernet v.2. These two standards can even coexist on the same cabling system, although devices using one standard cannot communicate directly with devices using the other. The MAC sublayer uses a type of contention access called Carrier Sense Multiple Access with Collision Detection (CSMA/CD). This technique reduces the incidence of collision by having each device listen to the network to determine whether it’s quiet (“carrier sensing”); a device attempts to transmit only when the network is quiescent. This reduces but does not eliminate collisions because signals take some time to propagate through the network. As devices transmit,

they continue to listen so they can detect a collision should it occur. When a collision occurs, all devices cease transmitting and send a “jamming” signal that notifies all stations of the collision. Then, each device waits a random amount of time before attempting to transmit again. This combination of safeguards significantly reduces collisions on all but the busiest networks.

IEEE 802.4

The 802.4 standard describes a network with a bus physical topology that controls media access with a token mechanism. This standard was designed to meet the needs of industrial automation systems but has gained little popularity. Both baseband and broadband (using 75-ohm coaxial cable) configurations are available. The workgroup for this set of standards is currently inactive.

IEEE 802.5 (Token Ring Network)

The IEEE 802.5 standard was derived from IBM’s Token Ring network, which employs a ring logical topology and token-based media access control. Data rates of 1, 4, and 16Mbps have been defined for this standard.

IEEE 802.6

The IEEE 802.6 standard describes a MAN standard called Distributed Queue Dual Bus (DQDB). Much more than a data network technology, DQDB is suited to data, voice, and video transmissions. The network is based on fiber-optic cable in a dual-bus topology, and traffic on each bus is unidirectional. When operated in pairs, the two buses provide a fault-tolerant configuration. Bandwidth is allocated by using time slots, and both synchronous and asynchronous modes are supported. The workgroup for this set of standards is currently inactive.

IEEE 802.7

This standard deals with integrating broadband solutions into a network environment. This standard is currently under development. The workgroup for this set of standards is currently inactive.

IEEE 802.8

This standard deals with methods of implementing fiber optic technology into networking environments. This standard is currently under development.

IEEE 802.9

The IEEE 802.9 standard supports a 10Mbps asynchronous channel, along with 96 64Kbps (6Mbps total bandwidth) channels that can be dedicated to specific data streams. The total bandwidth is 16Mbps. This standard is called Isochronous Ethernet (IsoEnet) and is designed for settings with a mix of bursty and time-critical traffic.

IEEE 802.10

This standard deals with security and encryption standards. This standard is currently under development.

IEEE 802.11

IEEE 802.11 is a standard for wireless LANs and is currently under development. A CSMA/CD method has been approved, but the final standard is pending.

IEEE 802.12

The IEEE 802.12 standard is based on a 100Mbps proposal promoted by AT&T, IBM, and Hewlett-Packard. Called 100VG-AnyLAN, the network is based on a star-wiring topology and a contention based access method whereby devices signal the wiring hub when they need to transmit data. Devices can transmit only when granted permission by the hub. This standard is intended to provide a highspeed network that can operate in mixed ethernet and token-ring environments by supporting both frame types.

IEEE 802.14

The 802.13 designation is not used, hence the last standard is known as 802.14. This standard is for transmitting data over cable TV lines. The committee is currently looking at a hybrid fiber/coax media. This is one of the up and coming areas for fast Internet access from a person’s home.